MINI FLASHLIGHT

User Guide

P RODUCT  C OMPONENTS

Back View

Bottom View

| KEY |  |  |  |
| --- | --- | --- | --- |
|  | Lens Power Button & Battery Level Indicator Body Poly Strap & Hang Loop |  | USB-C Port & Cover Magnetic Base & Battery Cap Li-ion 18650 ¦ 2600mAh Included USB-A to USB-C Cable Included |

*The Mini Flashlight arrives in demo mode.

To exit demo mode, simply charge the ﬂashlight via

the USB -C port or press and hold the power button

for 8-10 seconds.

OP ERA TING  INSTRUCTIONS

For optimal performance, charge the batteryfully beforeﬁrst use.

LIGHT SETTINGS

Press the power button to cycle through settings.

CLICK SET TING High Low Power O迁

OP ERA TING  INSTRUCTIONS

MANUAL BRIGHTNESS ADJUSTMENT

1.  While the ﬂashlight is ON, press and hold the power button to gradually increase or decrease brightness.

# 2.  Release the power button at the desired level.

CANDLE FLICKER MODE

With the light OFF, press and release the power button 3 times and press and hold the button a fourth time for approximately 3 seconds.

The ﬂashlight will enter Candle Flicker mode.

To exit, press the button again.

BA T TER Y  INSTALL A TION

1.  Turn battery cap counterclockwise to open battery slot.

2.  Insert Li-ion battery into battery slot with terminal end ﬁrst.

# 3.  Reattach battery cap by turning clockwise.

*Do not cross threads.

*Tighten securely to complete +/- connection.

FL ASHES

BA T TER Y  LEVEL  INDICA T OR  (BLI)

Battery Level Indicator (BLI) is located within the power button.

• When powering o迁, BLI light will quickly ﬂash indicating remaining battery power.

PO W ER LEVEL

75% - 100%

50% - 74%

25% - 49%

Less than 24%

• Ifthe ﬂashlight doesn’t turn on or BLI light ﬂashes red then o代, battery power level is too low and    must be recharged.

SOLID

Less than 24%

CHARGING  INSTRUCTIONS

With Li-ion battery installed, plug USB-C charging     cable into the Mini Flashlight’s USB-C port and other end into external power source to begin charging.

BLI light will ﬂash in a sequence correspoding with the battery level with approiximately 2 seconds between    each sequence.

PO W ER LEVEL

100%

75% - 99%

50% - 74%

25% - 49%

FL ASHES

FA Q

HOW DO I CLEAN THE LIGHT?

Use a slightly damp cloth to wipe down the Mini            Flashlight carefully. Do not use any chemical cleaners.

WILL THE RECHARGEABLE BATTERY EXPIRE?

Yes, all batteries will eventually expire and stop receiving a charge.

Barebones Li-ion batteries are available for purchase          at barebonesliving.com. Using batteries other than              Barebones Li-ion batteries will void the product warranty.

Li-ion batteries should not go in household garbage or  recycling bins. They can cause ﬁres during transport or at landﬁlls and recyclers. Instead, Li-ion batteries          should be taken to separate recycling or household        hazardous waste collection points.

WILL FINISH CHANGE OVER TIME?

By design, the Mini Flashlight ﬁnishes will change and creating a completely unique product. Over time, the   patina will slowly age, providing a beautiful, rich and   unique ﬁnish.

WARNING  &  SAFET Y

Do not submerge the ﬂashlight in liquid.

Keep product and battery out of reach of children.

Do not use or charge in temperatures below 32°F or above 113°F.

The warnings, cautions, and instructions in this manual cannot cover all possible conditions or situations that    could occur. Exercise common sense and caution when  using this light.

SP ECIFICA TIONS

XG2

LOW: 35 | HIGH: 150

~3000K

LI- ION 18650 | 2600mAh

LOW: 24+ HOURS | HIGH: 3+ HOURS

~6 HOURS

USB-C | 5V 0.5A

LED

LUMENS

TEMPERATURE

BATTERY

RUN TIME

CHARGE TIME

INPUT

For questions or concerns, contact

SUPPORT@BAREBONESLIVING.COM